250 Hour Service

Shire 90

| Boat Name: |  | Owner: |  |
| --- | --- | --- | --- |
| Location: |  |  |  |
| Engine Hours: |  | Date: |  |

| Description | Done | Comment |
| --- | --- | --- |
| Check operation of all controls |  | Lubricate key switch |
| Check and adjust all alternator belts |  |  |
| Change engine oil filter (See Materials Used) |  |  |
| Change engine oil (See Materials Used) |  |  |
| Change gearbox oil (See Materials Used) |  |  |
| Change primary fuel filter (See Materials Used) |  | Fit the fuel filter drain screw and seal from the old filter to the new filter |
| Change secondary fuel filter (See Materials Used) |  | Change annually |
| Check all fuel pipes are secure/no chaffing |  |  |
| Check security of all electrical connections |  | Check behind the panel |
| Visual inspection of all wiring |  |  |
| Check coolant levels |  |  |
| Check all hose clips for security |  |  |
| Check all cooling water pipes are secure/no chaffing |  |  |
| Check alignment of engine/shaft |  |  |
| Check propeller is secure |  |  |
| Check tightness of engine mounting bolts |  |  |
| Check tightness of drive coupling bolts |  |  |
| Visual check of stern gland |  |  |
| Check operation of bilge pump |  |  |

Materials Used

|  | Description | Part No. | Qty | Model |
| --- | --- | --- | --- | --- |
|  | Engine Oil 15W-40 20L Container | VC83070-020 |  | All |
|  | Gearbox Oil 10W40 5L Container | RDG6110 | 1 | All |
|  | Oil Filter | RE504836 | 1 | All |
|  | Primary Fuel Filter* | RDG9188346 | 1 | All |
|  | Secondary Fuel Filter* | RE62418 | 1 | All |

* Only filters which meet the Recreational Craft Directive should be fitted to your engine