50 Hour 1st Service

Shire 38/40/45/50

| Boat Name: |  | Owner: |  |
| --- | --- | --- | --- |
| Location: |  |  |  |
| Engine Hours: |  | Date: |  |

| Description | Done | Comment |
| --- | --- | --- |
| Check operation of all controls |  |  |
| Check and adjust all alternator belts |  |  |
| Change engine oil filter (See Materials Used) |  |  |
| Change engine oil (See Materials Used) |  |  |
| Change gearbox oil (See Materials Used) |  | Oil required dependant on gearbox fitted |
| Change primary fuel filter (See Materials Used) |  | N/A for Shire 38 |
|  |  | Fit the fuel filter drain screw and seal from the old filter to the new filter |
| Check all fuel pipes are secure/no chaffing |  |  |
| Check security of all electrical connections |  | Check behind the panel |
| Visual inspection of all wiring |  |  |
| Check coolant levels |  |  |
| Check all hose clips for security |  |  |
| Check all cooling water pipes are secure/no chaffing |  |  |
| Check alignment of engine/shaft |  |  |
| Check propeller is secure |  |  |
| Check tightness of engine mounting bolts |  |  |
| Check tightness of drive coupling bolts |  |  |
| Visual check of stern gland |  |  |
| Check operation of bilge pump |  |  |

Materials Used

|  | Description | Part No. | Qty | Model |
| --- | --- | --- | --- | --- |
|  | Engine Oil 10W40 5L Container (This oil is also used for PRM 150 & 280 Gearboxes) | RDG6110 | 2* | All |
| No Image | ATF Oil for PRM 90 Gearbox (If fitted) | ATF-1L | 1 | All |
|  | Oil Filter | 129150-35170 | 1 | All |
|  | Primary Fuel Filter | RDG9188346 | 1 | Shire 40 Shire 45 Shire 50 |

* A quantity of 2 is required even if a PRM 150 Gearbox isn’t fitted

** Only filters which meet the Recreational Craft Directive should be fitted to your engine